TT2400/TT5800 Firmware Release Notes

Oct 25, 2007

3.2.0-3.10.1(4M flash series)

| Version | Date | Change |
| --- | --- | --- |
| 3.2.0(SU) 3.2.1(AP) | Dec 13, 2005 | New feature: - Ping watchdog utility. - Replaced with new title picture Fixed: - Mac filter page error fixed - LED to show the right signal strength |
| 3.3.0(SU) 3.3.1(AP) (0112) | Jan 12, 2006 | New feature: - Introduced SDK 6.6 Fixed: - Ethernet Enhancement - WPA2 fixed - Reset button fix - Some Factory Default values, SSID(teletronics) - SNMP incomplete content fixed |
| 3.4.0(SU) 3.4.1(AP) | Mar 1, 2006 | New feature: - Automatically distinguish 2.4 and 5.8 radio card by showing different logo and radio type on webpage and EZ-Manager - Copyright Added Fixed: - Option for Antenna selection, default is “Use Antenna#1” - WPA2 fixed - Password login bug fix - Enhanced radio driver for noisy environment |
| 3.5.0(SU) 3.5.1(AP) | Mar 29, 2006 | New feature: - MAC control list increased to 24 clients for AP mode - Intra-BSS traffic blocking (Layer 2 Isolation) for AP mode - Register Status showed on the first page Fixed: - Signal Strength bug fix for EZ-Manager and SNMP - Title on Web GUI Changed to “Teletronics Wireless Device” |
| 3.6.0(SU) 3.6.1(AP) | June 9, 2006 | New feature: - Access Point supports up to 64 Clients - Client’s idle time is shown in AP’s client list - Number of clients associated is shown in AP’s client list - Reset to factory default through ezManager 3.0 (Activation key is needed for security reasons) - Roaming Enhancement by minimizing handover time in a separate firmware release Fixed: - SNMP Password/Community bug fixed(thank Dave) - SNMP system name, uptime and contact corrected(thank Dave) - Stop broadcasting “ICMP unreachable message” in different subnet (Critical/ thank Clay) - Timer overflow problem fixed(Critical/thank Dave) |
| 3.7.0(SU) 3.7.1(AP) | Aug 17, 2006 | New feature: - MAC control list increased to 64 clients for AP mode - No license key needed any more when changing firmware from AP to SU, vice versa. - BSSID association force feature(for SU only) Fixed: - Adjustable transmit power problem fixed - SNMP bug fix Note: Upgrading from version 3.6.x or older will lose previous configuration. |
| 3.8.0(SU) 3.8.1(AP) | Aug 28, 2006 | Fixed: - ACK_TIMEOUT bug fix, now it can support ack_timeout value over 255. - SNMP tx/rx traffic count |
| C3.9.0(SU) C3.9.1(AP) | Nov 3, 2006 | New feature: - Radio regional setting - Show Security info in AP lists - Show WDS Peer’s signal and info in assoc list - Support Atheros AR2413/2414 Radio Cards. - Limit Maximum output power to 23dBm for XA-622H and XG-622H high power radio card. Fixed: - Signal strength problem when SU is forced to associate to AP by BSSID - Support the AG-622 Radio card in 802.11a mode. - Remove the unsupported turbo channels (1103) Note: Upgrading from version 3.8.x or older will lose previous configuration. |
| C3.10.0(SU) C3.10.1(AP) | Feb 12, 2007 | New feature: - New LED/Signal Percentage mapping 5 LEDs: > 80%(-68dBm) 4 LEDs: > 60%(-75dBm) 3 LEDs: > 40%(-82dBm) 2 LEDs: > 20%(-88dBm) 1 LED: > 3%(-94dBm) 0 LED: No Signal(-95dBm) 2. Support 900MHz Radio card 3. Support Ubiquiti 900MHz radio w/ channel 4-5 only 4. Only scan designated channels for 900Mhz Radio 5. Output power XPD_GAIN patch for Zcom 900Mhz radio hw v0.6 6. Automated channel/region/Radio Mode/Turbo Mode/Rate Selection in Wireless Configuration Page 7. New LED/Signal Pattern for AP in WDS mode If AP is not associated with its WDS peer, the LED will enter searching mode If AP is associated with its WDS peer, the LED will show the actual signal level for the WDS link Fixed: - HTML form will no longer be cached in internet browsers - Disable SNMP by default for security reasons - Adjust super_mode/channel mapping for 4.9 radio - Force reboot after changing channel in start/main.c Note: Upgrading from version 3.8.x or older will lose previous configuration. |
| C3.11.0(SU) C3.11.1(AP) | Jun 13, 2007 | Fixed: - Delay the flash access to prevent configuration corruption in case of power outrage during the booting process. We recommend not powering cycle the device for at least 30 seconds during booting process or changing configuration settings. Note: Upgrading from version 3.10.x or higher will NOT lose previous configuration. But software “Reset Configuration” is still recommended, 1 minute after firmware is upgraded, before doing any configuration to the Units. Especially if firmware is upgraded from version 3.9.x or lower. |
| C3.14.0(SU) C3.14.1(AP) | Oct 25, 2007 | New feature: - Password is needed to change IP address and upgrade firmware through EzManager 3.3 for security protection - SU now supports QoS rate control for both wireless uplink and downlink - AP now supports bandwidth rate control for all wireless clients - Add region 26 with all available channels, please use with caution. - Add support for several wireless radio Fixed: Note: Upgrading from version 3.11.x or older will lose previous configuration. |

3.1.0-3.1.101(2M flash series)

| Version | Date | Change |
| --- | --- | --- |
| 3.1.2(SU) 3.1.3(AP) | Oct 10, 2005 | Fixed: some web interface bug: - Admin page: DHCP-->DHCP Client. - APs page(bridge only): singal strength --> signal strength. - Wireless: rx output power(mw)-> rx output power. |
| 3.1.4(SU) 3.1.5(AP) | Jan 13, 2006 | New feature: - Ping watchdog utility. - VLAN transparent support for SU Fixed - SNMP incomplete content fixed - MAC filter page error fixed |
| 3.1.40(SU) 3.1.41(AP) | Mar 7, 2006 | Same as version 3.4.0/3.4.1. |
| 3.1.50(SU) 3.1.51(AP) | Mar 29, 2006 | Same as version 3.5.0/3.5.1. |
| 3.1.60(SU) 3.1.61(AP) | June 9, 2006 | Same as version 3.6.0/3.6.1. |
| 3.1.70(SU) 3.1.71(AP) | Aug 17, 2006 | Same as version 3.7.0/3.7.1. |
| 3.1.80(SU) 3.1.71(AP) | Aug 28, 2006 | Same as version 3.8.0/3.8.1. |
| C3.1.80(SU) C3.1.81(AP) | Nov 3, 2006 | Same as version 3.9.0/3.9.1. |
| C3.1.100(SU) C3.1.101(AP) | Feb 12, 2007 | Same as version 3.10.0/3.10.1. |
| C3.1.110(SU) C3.1.111(AP) | Jun 13, 2007 | Same as version 3.11.0/3.11.1. |
| C3.1.140(SU) C3.1.141(AP) | Oct 25, 2007 | Same as version 3.14.0/3.14.1. |

EzManager Utility Release Notes

June 9, 2006

| Version | Date | Change |
| --- | --- | --- |
| 3.0 | June 9, 2006 | New feature: - Support reset factory default remotely - Support reboot remotely - Clear ARP table before searching - New logo and icon |